Yep—looking at this frame, your plan is absolutely feasible.

What I see (and why it’s workable)

- Strong circular edges on most balls → good candidates for Hough + contour circularity.

- Many are hollow (donut-like) → our “annulus handler” (outer+inner edge pairing) will recover the outer radius reliably.

- There’s specular glare from the clear cover and lighting → manageable with bilateral filtering + CLAHE and by preferring edge strength over raw intensity.

- Crowding/occlusion near the bottom → contour path + NMS will reduce duplicates; tracking will stabilize IDs.

- The purple ring + bolt are static and easy to mask out so they don’t create false circles.

Concrete tips for this capture setup

- Add a circular ROI mask to exclude the purple flange and center void before detection.

- If possible, put a 50 mm 2-point sticker on the clear cover for 1–2 seconds at the start → gives a quick click-to-calibrate px/mm.

- To minimize blur at ~60 fps: shutter ~1/120 s (or faster) with extra light.

What this means for your demo

- Live toggling of 4 / 6 / 8 / 10 mm overlays over this exact footage will be smooth—the detections are cached; toggles only re-draw.

- Hollow beads will still color correctly because classification uses the outer diameter.

Two quick confirmations:

- Can you add the two-point (50 mm) sticker for quick, repeatable calibration on future recordings? (Default: yes.)

- Are all balls in the demo hollow, or a mix of hollow + solid? (Default: mix; we’ll keep annulus handling on.)

###  our current architecture ###

SPEC-01-Mill Simulation – Video Color Coding

Background

Molycop (mining consumables manufacturer) provided a grinding-mill simulation video showing mixed steel balls of multiple diameters tumbling inside a mill. The project—initiated by Eduardo Nunez as a personal initiative (not formally company-sponsored)—aims to analyze those videos and overlay a color map that distinguishes each ball by size. The system will be used for educational/analysis purposes to quickly visualize size distribution and behavior inside the rotating mill.

Context gathered so far:

- Stakeholders: Eduardo Nunez (VP, Process Optimization) and Michael Hahn (Sr. Applications Engineer).

- Platform: Windows desktop environment.

- Input: pre-recorded mill simulation videos (40–50 RPM typical), shiny metallic media with through-holes in some balls; varying lighting and motion blur.

- Initial size groups discussed: 4 mm, 6 mm, 8 mm, 10 mm (awaiting confirmation).

- Constraints & risks: motion blur, occlusions, metallic reflections/holes complicating edges, need for pixel-to-mm calibration based on camera distance, limited time for dataset labeling.

- Current status: team pivoted from ML labeling (e.g., YOLOv8) to a classical computer-vision approach (OpenCV) emphasizing contrast, circularity detection, and post-processing. Cache per-frame detections to support UI toggles and fast overlays. Target accuracy ~80–90% under stable lighting and moderate RPM.

Assumptions (to confirm):

- Videos are single-camera, fixed viewpoint, constant frame rate; minimum 1080p.

- Output is an annotated video (e.g., MP4) plus optional per-frame JSON/CSV of detections.

Requirements

Must Have

- Detect all visible balls per frame and classify into 4 size bins (4, 6, 8, 10 mm) using a pixel→mm calibration step.

- Overlay color codes by class on the video and export an annotated MP4.

- Export per-frame CSV/JSON with: frame_id, centroid (x,y), estimated_diameter_mm, class, confidence.

- Configurable bin thresholds (mm) and calibration factor (px/mm) via a YAML/GUI control.

- Process 1080p clips offline at ≥10 FPS on a mid-range Windows laptop (e.g., i5/16GB, no GPU assumed for MVP).

- Robust to common artifacts: specular highlights, partial occlusions, through-holes, moderate motion blur.

- Simple GUI: load video → set calibration from a known distance/object → run → preview overlay → export results.

Should Have

- Batch processing of multiple videos; auto-save session config.

- Per-clip summary stats and histograms (counts per class, per-frame density, % area coverage).

- Optional light-weight tracking for temporal smoothing (ID persistence across frames) to reduce flicker.

- Internal accuracy goal (≥85% F1 on a small validation subset) for engineering only; no accuracy numbers shown in UI.

Will Not Have (MVP)

- Multi-camera 3D reconstruction, physics/DEM coupling, or contact force estimation.

- Cloud backend or large-scale data lake; strictly local processing.

- Guarantees under extreme RPM/high blur conditions.

UI Behavior (MVP)

- Minimal flow: Load video → Set calibration (px/mm) → Process → Auto-load result → Preview/Export.

- After processing, the annotated result is auto-loaded into the player.

- The player supports real-time toggles for size classes (4/6/8/10 mm) while the video is playing; overlays appear/disappear instantly without reprocessing.

- Export options: Save annotated MP4 honoring the current toggle state (user choice captured at export time). Also allow saving the raw detections (CSV/JSON) for later re-rendering.

- No accuracy metrics or model diagnostics are displayed.

Method

High-Level Architecture

- Frontend (Desktop UI): Python PySide6/Qt video player with real-time overlay toggles (4/6/8/10 mm).

- Processing Core: Python OpenCV pipeline (CPU-only MVP) with optional NumPy/scikit-image helpers; multiprocessing for faster-than-realtime when available. Results are cached per frame for instant toggling.

- I/O: OpenCV VideoCapture/VideoWriter (MP4/H.264, source FPS/resolution), plus CSV/JSON of detections. Calibration and class thresholds stored in YAML.

- File Layout: one working folder per clip containing config.yaml, detections.jsonl, overlay.mp4.

Calibration

Default: Ring-based auto-calibration (no sticker needed)

- Measure once the true inner diameter of the blue ring (in millimeters).

- The app detects the ring via Hough → gets pixel_diameter.

- Compute px_per_mm = pixel_diameter / true_mm_diameter.

- Advantages: repeatable, no parallax/plane offset, fast.

Fallback: Two-point ruler (sticker/mini-ruler)

- Place two markers 50.0 mm apart briefly in the same plane as the beads.

- Click both points in the UI → px_per_mm = pixel_distance / 50.0.

- Use when the ring is occluded or cannot be measured once with calipers.

Optional: Known-ball fit

- Fit a circle to a clearly visible bead; enter its known diameter D_known.

- px_per_mm = (2 * r_px) / D_known.

Bins are configurable (default ±1 mm around each nominal size):

- 4 mm: [3.0, 5.0)

- 6 mm: [5.0, 7.0)

- 8 mm: [7.0, 9.0)

- 10 mm: [9.0, 11.0)

Detection & Classification Pipeline (per frame)

- Preprocess

  - Convert to grayscale.

  - Denoise with bilateral filter (preserves edges, reduces specular noise).

  - Contrast normalize with CLAHE (mitigate lighting/reflection swings).

  - Optional temporal median blend of previous N frames to stabilize background.

- Adaptive Thresholding & Edge Extraction

  - Compute edges with Canny (auto-tuned via median heuristic).

  - Morphological close→open to remove small holes/sparkle artifacts.

- Candidate Circle Proposals

  - Dual strategy for robustness:
a) Circular Hough Transform (cv2.HoughCircles) across a narrow radius range per class (derived from px/mm and tolerances).
b) Contour analysis: find contours → filter by circularity (4π·Area/Perimeter²), solidity, minEnclosingCircle fit error.

- Handle Hollow/Through-hole Balls

  - Detect annulus (donut) shapes via inner/outer edge pairs; compute outer radius as ball radius.

  - If inner edge present, ignore center hole when computing circularity/area.

- Non-Maximum Suppression (NMS)

  - Merge duplicates from Hough and contour paths using IoU/center distance thresholds.

- Diameter & Class

  - Convert radius_px→diameter_mm using calibration.

  - Assign class via configurable bins (defaults: [3–5), [5–7), [7–9), [9–11) mm).

  - Store confidence as a weighted score of: edge strength, fit residual, circularity.

- (Optional) Short-term Tracking

  - Lightweight Kalman filter + nearest-neighbor to stabilize IDs and reduce flicker; purely for smoothing, not required for toggles.

- Cache

  - Persist FrameDetections {frame_id, [Ball{x,y,r_px,diameter_mm,class,conf}]} to JSONL and in-memory LRU for immediate overlay.

Overlay Rendering

- For each frame during playback, draw color-coded circles/labels for enabled classes only.

- Toggle changes do not trigger re-detection—only re-render from cache.

- Export uses same renderer honoring current toggle state.

ROI Mask Tool (one-time per clip/site)

Purpose: Ignore bolts, flange, hands, wheels; keep detections inside the working mill area.

Workflow:

- Pause on a clear frame → open ROI Tool.

- Auto-Circle: Hough finds the inner ring; user can nudge radius/center and set an inner margin (e.g., 10–20 px).

- (Optional) Draw Exclusion Polygons for any remaining static intrusions.

- Save → writes roi_mask.png (white = valid, black = excluded); reused automatically.

Runtime: candidates with centers outside white are discarded before any scoring.
Bonus: a low-motion background check rejects persistent edges even inside the mask.

Repo

mill_presenter/

src/mill_presenter/

app.py

models.py

ui/

main_window.py          # QMainWindow + player + toggles (4/6/8/10)

calibrate.py            # ring-based auto-cal + two-point fallback

roi_tool.py             # auto-circle + polygon mask -> roi_mask.png

widgets.py

core/

playback.py             # Frame Loader: PyAV decode (NVDEC→CPU fallback)

orchestrator.py         # one-time detection pass orchestration

processor.py            # OpenCV pipeline (stub, then fill)

overlay.py              # QPainter overlays (shared by player/exporter)

exporter.py             # Raw / With Overlays export (H.264)

cache.py                # JSONL I/O + RAM ring buffer (~240 frames)

nms.py                  # non-maximum suppression helpers

annulus.py              # through-hole handling (inner/outer)

classify.py             # px->mm + binning

utils/

config.py               # YAML load/save + defaults

logging.py              # rotating log

paths.py

configs/

sample.config.yaml

exports/

data/                       # (optional) sample clip & dummy detections

scripts/

setup.ps1  run.ps1  build.ps1  bench.ps1

tests/

test_classify.py  test_nms.py  bench_toggle_latency.py

pyproject.toml

.env.example

README.md